SmartButton User's Manual

1. Hardware Overview

- UP Button: Controls the device to move up/open.

- DOWN Button: Controls the device to move down/close.

- RESET Button: Used for system reset or factory reset.

- LED Indicator: Includes Green and Red indicators to show status and battery level.

2. Basic Control Functions

Curtain/Device Control

- Open/Move to Top:

  - Operation: Single-click the UP Button.

  - Function: Moves the paired device to the 0% position (Top Position).

- Close/Move to Bottom:

  - Operation: Single-click the DOWN Button.

  - Function: Moves the paired device to the 100% position (Bottom Position).

Battery Level Indication

Each time any button (UP/DOWN/RESET) is pressed, the LED will light up to indicate the current battery level:

- Green ON: Sufficient battery (Battery Voltage > 2.7V).

- Red ON: Low battery (Battery Voltage ≤ 2.7V), please consider replacing the battery.

3. Pairing and Setup

Entering Pairing Mode

- Operation: Press and hold both the UP Button and DOWN Button simultaneously.

- Indication: Until the Green light starts flashing rapidly.

- Description:

  - The remote will enter scanning and pairing mode, which lasts for about 3 minutes (180 seconds).

  - Pairing Successful: The Green light will stay on for 2 seconds (Hold).

  - Pairing Timeout (No Device): The Red light will flash 6 times.

  - Pairing Rejected/Error: The Red light will flash 4 times.

Exiting Pairing Mode

- In pairing mode, single-click any button (UP or DOWN) to immediately terminate pairing mode, and the LED will turn off.

4. System Maintenance

System Reset (Soft Reset)

- Operation: Single-click the RESET Button.

- Function: The remote control will restart (System Reset).

- Indication: The Green or Red light will also briefly turn on according to the battery level.

Factory Reset

- Operation: Press and hold the RESET Button.

- Indication: The Green light will stay on.

- Function: Clears all bound device information (deletes all Peers), and the remote control is restored to its initial state.

5. LED Status Quick Reference Table

| Status | LED Behavior | Description |
| --- | --- | --- |
| Button Operation | Green light on | Operation valid and battery sufficient |
|  | Red light on | Operation valid but battery low |
| Pairing | Green light flashes rapidly | Scanning for devices (Pairing mode) |
| Pairing Result | Green light on for 2 seconds | Pairing successful |
|  | Red light flashes 6 times | Pairing timed out/No device found |
|  | Red light flashes 4 times | Pairing rejected or error |
| Factory Reset | Green light stays on | Data has been cleared |

6. RF and Electrical Parameters

| Parameter | Specification |
| --- | --- |
| Operating Frequency | 2402 – 2480 MHz |
| Rated Power | 1.5 W |
| Rated Voltage | 3.0V |
| Rated Current | 500 mA |
| Operating Temperature | 0–40 °C |