How to Make a J-LINK by Raspberry Pi Zero

- Set up Raspberry Pi 4

  - Download Raspberry Pi Imager from https://www.raspberrypi.com/software/

  - Download the image from https://raspi.debian.net/daily/raspi_4_bullseye.img.xz

  - Flash Debian TF card

  - Edit /boot/config.txt and add ‘enable_uart=1’ and ‘gpu_freq=360 ’to the end of the file, and save.

  - Login by user: root, no password

  - apt-get update

  - apt-get install net-tools

  - apt-get install nano vim screen

  - apt-get install openssh-server telnetd telnet

  - Edit /etc/ssh/sshd_config

- nano /etc/ssh/sshd_config

- edit 'PermitRootLogin AUTH' to 'PermitRootLogin yes'

  - service ssh restart

  - apt-get install samba

  - smbpasswd -a                (def:123456)

  - mkdir /home/shared

  - nano /etc/samba/smb.conf

- #add this to the very end of the file:

- [shared]

- path = /home/shared

- valid users = root

- read only = no

  - service smbd restart

  - testparm

  - passwd root		(def: 123456)

- JTAG Pinout

- Install and config OpenOCD (Also see: https://neucrack.com/p/142)

  - sudo apt-get update

  - sudo apt-get install -y autoconf libtool libftdi-dev libusb-1.0-0 libusb-1.0-0-dev git

  - sudo apt-get install gpiod libgpiod-dev build-essential pkg-config libtool

  - git clone --recursive git://git.code.sf.net/p/openocd/code openocd-code

  - cd openocd-code

  - ./bootstrap

  - ./configure --enable-sysfsgpio\

--enable-linuxgpiod \

--enable-maintainer-mode \

--disable-werror \

--enable-ft2232_libftdi \

--enable-ep93xx \

--enable-at91rm9200 \

--enable-usbprog \

--enable-presto_libftdi \

--enable-jlink \

--enable-vsllink \

--enable-rlink \

--enable-arm-jtag-ew \

--enable-dummy \

--enable-buspirate \

--enable-ulink \

--enable-presto_libftdi \

--enable-usb_blaster_libftdi \

--enable-ft2232_libftdi\

--prefix=/usr

--disable-dependency-tracking

  - make

  - make install

  - git clone https://github.com/Neutree/MT7688-OpenOCD   (download the cfg file from Angelic47)

  - Edit jlink_gpio.cfg for JTAG I/Os

adapter driver linuxgpiod

transport select jtag

linuxgpiod gpiochip 0

linuxgpiod jtag_nums 23 26 12 3

linuxgpiod trst_num 17

linuxgpiod srst_num 19

adapter srst delay 100

jtag_ntrst_delay 100

reset_config trst_and_srst

  - openocd -f jlink_gpio.cfg

  - telnet localhost 4444

  - load_image cc.mp4 0x88000000

- Reference: GPIO commands example

- gpiodetect		# detect GPIO devices

- gpioinfo 0		# check the configuration of GPIO device 0

- gpioset 0 6=1		# set the GPIO line 6 of the gpio controller 0

- gpioget 0 6		# get the GPIO line 6 of the gpio controller 0

- gpiomon 0 6		# print out the behavior of  GPIO line 6 of the gpio controller 0

- Use Bus Priate to program SPI flash

- apt-get install flashrom

- flashrom -p buspirate_spi:dev=/dev/ttyUSB0,spispeed=8M -r /home/shared/MT7688-OpenOCD/backup.bin

- flashrom -p buspirate_spi:dev=/dev/ttyUSB0,spispeed=8M -w /home/shared/MT7688-OpenOCD/O2S_entire_img.bin

- flashrom -p buspirate_spi:dev=/dev/ttyACM0,spispeed=8M -r /home/shared/MT7688-OpenOCD/backup.bin

- flashrom -p buspirate_spi:dev=/dev/ttyACM0,spispeed=8M -w /home/shared/MT7688-OpenOCD/O2S_entire_img.bin

- Wiring of hardware

| Signal Name | MT7688 Pin Name | O2S Pin Name |
| --- | --- | --- |
| TDO | EPHY_LED0, 143 |  |
| TDI | EPHY_LED1, 142 |  |
| TMS | EPHY_LED2, 141 |  |
| TCK | EPHY_LED3, 140 |  |
| TRST | EPHY_LED4, 139 |  |

- rsv