How to Make a J-LINK by Raspberry Pi Zero

- Set up Raspberry Pi Zero

  - Download Raspberry Pi Imager from https://www.raspberrypi.com/software/

  - Flash Debian TF card

  - Edit /boot/config.txt and add ‘enable_uart=1’ to the end of the file, and save.

  - Login by user: pi, pass: raspberry

- Set up ethernet extension board

  - Connect a Raspberry Pi Zero to an ethernet extension board, pinout as below

Take an ethernet extension PCB from Amazon

(https://www.amazon.ca/gp/product/B087TD33KM/ref=ppx_yo_dt_b_asin_title_o00_s00?ie=UTF8&psc=1)

as example:

| Signal name | Raspberry Pi Zero | Ethernet Extension |
| --- | --- | --- |
| SCK | Pin 23(GPIO14) | Pin 16 |
| MISO | Pin 21(GPIO13) | Pin 15 |
| MOSI | Pin 19(GPIO12) | Pin 14 |
| SS | Pin 24(GPIO10) | Pin 13 |
| INT | Pin 22(GPIO6) | Pin 4 of ENC28J60 on PCB |

  - Edit /boot/config.txt and add ‘dtoverlay=enc28j60’ to the end of the file, and save.

  - Enable SPI as below:

#sudo raspi-config

Go to Interface options, enable SPI

  - Assign a static IP

sudo nano /etc/dhcpcd.conf

Add:

interface eth0

static ip_address=192.168.1.142/24

static routers=192.168.2.1

static domain_name_servers=192.168.2.1

  - Reboot

- JTAG Pinout

- Install and config OpenOCD (Also see: https://neucrack.com/p/142)

  - sudo apt-get update

  - sudo apt-get install -y autoconf libtool libftdi-dev libusb-1.0-0 libusb-1.0-0-dev git

  - sudo apt-get install gpiod

  - sudo apt-get install libgpiod-dev

  - sudo apt-get install git

  - git clone --recursive git://git.code.sf.net/p/openocd/code openocd-code

  - cd openocd-code

  - ./bootstrap

./configure --enable-sysfsgpio\

--enable-linuxgpiod \

--enable-maintainer-mode \

--disable-werror \

--enable-ft2232_libftdi \

--enable-ep93xx \

--enable-at91rm9200 \

--enable-usbprog \

--enable-presto_libftdi \

--enable-jlink \

--enable-vsllink \

--enable-rlink \

--enable-arm-jtag-ew \

--enable-dummy \

--enable-buspirate \

--enable-ulink \

--enable-presto_libftdi \

--enable-usb_blaster_libftdi \

--enable-ft2232_libftdi\

--prefix=/usr

  - make

  - sudo make install

  - git clone https://github.com/Neutree/MT7688-OpenOCD   (download the cfg file from Angelic47)

  - Edit jlink_gpio.cfg for JTAG I/Os

adapter driver linuxgpiod

transport select jtag

# tck tms tdi tdo

linuxgpiod gpiochip 0

linuxgpiod jtag_nums 23 26 12 3

linuxgpiod trst_num 17

linuxgpiod srst_num 19

adapter srst delay 100

jtag_ntrst_delay 100

reset_config trst_and_srst

  - Edit jlink_gpio.cfg as promoted after sudo openocd -f jlink_gpio.cfg

  - sudo openocd -f jlink_gpio.cfg

  - telnet localhost 4444

- Check the availability of 	GPIOs

gpiodetect		# detect GPIO devices

gpioinfo 0		# check the configuration of GPIO device 0

- Install telnet

sudo apt-get  install telnetd

- Enable ssh

  - Go into /boot

  - sudo touch ssh

  - Reboot

  - sudo raspi-config

  - Goto ‘interface options->ssh->enable it’

- Telnet or ssh from another windows computer by PUTTY.

- Sftp from another window computer by PSFTP

  - lcd C:\Users\bin\Downloads

- Rsv

- Wiring of hardware

| Signal Name | MT7688 Pin Name | O2S Pin Name |
| --- | --- | --- |
| TDO | EPHY_LED0, 143 |  |
| TDI | EPHY_LED1, 142 |  |
| TMS | EPHY_LED2, 141 |  |
| TCK | EPHY_LED3, 140 |  |
| TRST | EPHY_LED4, 139 |  |

- rsv